|  | Mediation and Mediation and billing sub system Function Requirement Specification |  |  |  |
| --- | --- | --- | --- | --- |
|  | Issue 01 |  |  |  |
|  | Date 2014-01-23 |  |  |  |
|  | HUAWEI TECHNOLOGIES CO., LTD. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2013. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute the warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | http://www.huawei.com |
| Email: | support@huawei.com |

About This Document

Intended Audience

The CDR-based requirement of mediation and mediation and billing sub system for project is described in this document. The third party software provider develop this sub system according this document after confirm the requirement.

Revision History

| Date | Version | Author | Description |
| --- | --- | --- | --- |
| 2014-01-09 | 1.0 | zhanglichun | Initail Draft |
| 2014-01-13 | 1.1 | zhanglichun | Modified |
| 2014-01-23 | 1.3 | zhanglichun | Modified |
| 2014-02-07 | 1.4 | zhanglichun | Add IDD rate |

References

The following table lists references mentioned used while creating this document. The referenced documents can be found on the SharePoint location for the BTCL Project.

| Nr. | Title | Filename | Version |
| --- | --- | --- | --- |

Stakeholders

The following table lists the document stakeholders in Subcontractor and Huawei.

| Subcontrator |  |  |  |
| --- | --- | --- | --- |
| Name | Role | Contact Details |  |
| Huawei |  |  |  |
| Name | Role |  | Contact Details |
| Weizhonggui | SR |  | weizhonggui@huawei.com |
| Mohammad Mosabbir Rana | PD |  | mosabbir.rana@huawei.com |
| Zhanglichun | IMS SE |  | jonny.zhanglichun@huawei.com |

Abbreviations

| Abbreviation | Full Form |
| --- | --- |
| IGWB | iGateway Bill |
| NGN | Next Generation Network |
| CDR | Call Detail Record |
| FTP | File Transfer Protocol |
| NE | Network Element or Network Equipment |
| SFTP | Secured File Transferred Protocol |

Contents

About This Document	iii

1	Introduction	9

1.1	Description	9

1.2	Interface to IGWB	9

1.2.1	Overview	9

1.3	Interface to Billing center	10

1.3.1	Overview	10

1.4	Scope	10

1.5	Open point and Risk	10

2	Business Function For BILLING	11

2.1	MEDIATION AND BILLING SUB SYSTEM	11

2.1.1	Requirement Reference	11

2.1.2	Definition	11

2.1.3	Assumptions	11

2.1.4	Functional Requirement	11

2.1.5	Business Rule	11

2.1.6	Implementation	12

2.2	Maintain and Configuration	20

2.2.1	Configuration	20

2.3	Performance Requirment	20

Appendix A – Glossary	21

# Introduction

## Description

Project mediation and billing sub system is described in this document. This mediation and billing sub system is only for CDR-based charging in project. Project mediation and billing sub system is integrated platform to collect usage data from the IGWB and send the data to existing billing center. It include collecting, charging, reporting, billing, configuring and browsing. User center Design easy to use and maintain with table instead configuring in the file and browse CDR. Perfomance is very high and flexible solution to satisfy the requirement faster.

## Interface to IGWB

### Overview

In Bangladesh the project, the mediation and billing sub system collect binary cdr file generated by IGWB by FTP protocol. IGWB is redundancy and there are two actural ip address. Mediation and billing sub system need to connect with two IGWB server and receive the billing file from IGWB. If IGWB1 has broken down, IGWB2 will push billing file to mediation and billing sub system. Mediation and billing sub system should process 1M(1000,000) CDRs per day.

## Interface to Billing center

### Overview

Mediation and billing sub system will push billing file to billing center in CSV format by ftp protocol monthly. Billing center will add additional information to construct a new invoice for priting and charging. The additional information include subscriber name, address, and other information etc.

## Scope

This section provides basic information about NE’s mediation and billing sub system application

| S. No | UpStream |  | DownStream |  |
| --- | --- | --- | --- | --- |
|  | Name | Collection Protocol | Name | Dispatch Protocol |
| 1 | IGWB` | FTP | Billing Center | FTP |

## Open point and Risk

- No

# Business Function For BILLING

## MEDIATION AND BILLING SUB SYSTEM

### Requirement Reference

Function list ID: F_Billing_IGWB

### Definition

Mediation and billing sub system need to collect the cdrs from IGWB and need to distribute to Billing Center.

### Assumptions

The following table lists the assumptions that were made in writing this function.

| Assumption ID | Description |
| --- | --- |
| F_Billing_IGWB | Output file format and description is not confirm.it assumed that mediation and billing sub system will send the text format. |

### Functional Requirement

The following table describes the functional requirements.

| Requirements ID | Requirements Description | Business Rule | Priority |
| --- | --- | --- | --- |
| F_Billing_IGWB _001 | Mediation and billing sub system collect the CDRs from the IGWB |  |  |
| F_Billing_IGWB_002 | Mediation and billing sub system process the cdrs for Billing Center |  |  |
| F_Billing_IGWB _003 | Mediation and billing sub system Need to encode the CDRs as per the Downstream Format. |  |  |
| F_Billing_IGWB _004 | Mediation and billing sub system Need to upload the CDRs to Billing Centre. |  |  |

### Business Rule

NA

### Implementation

#### Business flow for mediation and billing sub system

#### Collection

Collection is the process of collecting CDR files from the network elements and making the data available for further business processing. Mediation and billing sub system needs to receive the cdr’s from NE using FTP protocol.

Mediation and billing sub system directly receives CDR files from IGWB through standard FTP protocol. IGWB CDR processing scheme is time or volume based (Example, CDR file is generated at specified time interval or upon reaching a predefined file size).

- Input Filename Format

File name :

The IGWB converts the received original CDRs and fills the contents in a final CDR file.

In IGWB, customer can configure the name of final CDR file based on site requirements. Generally, the final CDR file is named in the format of Start identifier of the final CDR + Serial number of the final CDR + final CDR file name extension. The configuration is finished in IGWB server.

- Start identifier of the final CDR: It is configured during configuration of a CDR storage object and generally set to b.

- Serial number of final CDR: It is configured during configuration of a CDR storage object. The final CDRs are sequentially numbered in one channel.

- The final CDR file name extension: It is configured during configuration of a CDR storage object and generally set to .dat.

For Example:

b00002627.dat

- Collection Configuration Properties

Below table is for task of remote collection

| FTP | Parameter Name | Description | Values |
| --- | --- | --- | --- |
|  | Ip address1 | Address of Remote NE1 | <TBD> |
|  | Login username1 | Username of remote NE1 | <TBD> |
|  | Login password1 | Login password of remote NE1 | <TBD> |
|  | Ip address2 | Address of Remote NE2 | <TBD> |
|  | Login username2 | Username of remote NE2. | <TBD> |
|  | Login password2 | Login password of remote NE2 | <TBD> |
|  | Remote directory | Path of remote directory for collect the files | <TBD> |
|  | Remote File Storage Path | Path for backing up remote files after CDR collection. | E:/backup/second/detail |
|  | Local File Storage Path | Relative path for storing local files in mediation and billing sub system |  |
|  | Local File Storage Name | Name of a local file. | Original file name |
|  | Scan subdirectories | Whether to scan and collect the files in subdirectories. | YES |
|  | Uncompress | Flag indicating whether to decompress the CDR files to be collected. | NO |

- Collection Checking

Mediation and billing sub system receives CDR files from IGWB. Maybe CDR files are lost or repeated. Mediation and billing sub system need to check this situation and get correct CDR files by checking the sequence number of file name.

#### Decoding

Decoder element will decode the CDR files into internal format for further processing. The record type are

- If decoding is successful, the RAW CDR file will be stored in backup directory (configurable).

- If decoding is failed, the RAW CDR file will be stored in error CDR directory (configurable).

Internal format of decoded file is not stored anywhere in the system (file or database), and it’s kept in memory for further processing (filter, validation, conversion, encoder)<TBD>.

Below parameters are configurable.

| Parameter Name | Description | Values |
| --- | --- | --- |
| Cdr Backup Directory | Directory for backing up the raw CDRs that are successfully decoded. The following two path formats are supported: - Full path: For example, /home/prm/cdr/bak/tapin/orig_cdr. - Relative path: For example, ~/cdr/bak/tapin/orig_cdr. NOTE: A relative path is supported only when basepath is correctly set in the config.ini file. If the parameter is set, it indicates that RAW CDRs are backed up to the backup directory after decoding; if the parameter is not set, it indicates that RAW CDRs are deleted after decoding. |  |
| Error CDR directory | Directory for storing the erroneous CDRs that fail to be decoded. The following two path formats are supported: - Full path: For example, /home/prm/cdr/err/tapin/orig_cdr. - Relative path: For example, ~/cdr/err/tapin/orig_cdr. NOTE: A relative path is supported only when base path is correctly set. |  |
| Decoding the last file | Indicates whether to decode the last CDR file. There will be Drop down box need to select Yes/No. Default Value is Yes.Incase Yes will help component to start the file from where decoding need to be started. |  |
| Whether skip exception records when decode failed | Mode of processing exception CDRs when CSV CDR files are decoded. The options are as follows: Yes If a CDR fails to be decoded, the system moves the CDR to the error CDR directory, continues to decode subsequent CDRs, and generates an log displaying the number of unsuccessfully decoded CDRs. No If a CDR fails to be decoded, the system stops the decoding, moves the CDR to the error CDR directory, and generates an alarm. | Yes |

- File Format

RAW CDRs are in binary format. The format is as clause 1.2 of attatched file. Please find the below attach sample CDR File.

#### Error Record Detection

Error record check will be performed for all CDRs based on the error key fields. The error CDRs are stored in error CDR directory and not be considered for normal processing flow.

Following key fields are used for error check.

| No | Field | Value |
| --- | --- | --- |
| 1 | The length of billing file | Must be 907 times, such as 907, 1804, 2712 etc. |
| 2 | Conversation duration | < 3 day |
| 3 | Valid indicator | 0 |
| 4 | Answering time | Year: >2013 and <2050,month: 1~12, day: 1~31, hour:1~23, minute:1~59, second: 1~59 |
| 5 | Call end time | Year: >2013 and <2050,month: 1~12, day: 1~31, hour:1~23, minute:1~59, second: 1~59 |
| 7 | Caller number | Must be in the configured number segment |

#### Conversion rule

Data conversion is the most important function of the mediation and billing sub system. The purpose of conversion is to generate the data required by downstream business systems.

The CDR conversion in the mediation and billing sub system is a rule-based component. The rule script can be customized.

The binary detail ticket come from IGWB and processed by mediation and billing sub system to generate a user billing file in CSV format.

The output file should include these fields.

| ID | Field | Null? | Type | allignment | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | Subscriber num | NOT NULL | CHAR(15) | Right allignment | AC+SN, example:28888888 |
| 1 | BTCL-BTCL unit | NOT NULL | Interger(14) | Right allignment | Refer to clause 2.1.6.5 A |
| 2 | BTCL-other unit | NOT NULL | Interger(14) | Right allignment | Refer to clause 2.1.6.5 A |
| 3 | Num of IDD | NOT NULL | Interger(14) | Right allignment | The count of IDD call. The prefix of called number of IDD call begin with ‘00’ or ‘012’. |
| 4 | Future use | NULL | CHAR(15) | Right allignment | FILL WITH BLANK |
| 5 | Future use | NULL | CHAR(15) | Right allignment | FILL WITH BLANK |
| 6 | Future use | NULL | CHAR(15) | Right allignment | FILL WITH BLANK |
| 7 | BTCL-BTCL fee | NOT NULL | NUMBER(14,2) | Right allignment | =(ID1*0.1)TK, example: BTCL-BTCL call unit is 6, then BTCL-BTCL fee is 0.6TK. |
| 8 | BTCL-other fee | NOT NULL | NUMBER(14,2) | Right allignment | =(ID2*0.8)TK, example: BTCL-other call unit is 6, then BTCL-other fee is 4.8TK |
| 9 | IDD fee | NOT NULL | NUMBER(14,2) | Right allignment | Refer to clause 2.1.6.5 B |
| 10 | Line rent | NOT NULL | NUMBER(14,2) | Right allignment | 160 TK, configurable |
| 11 | total | NOT NULL | NUMBER(14,2) | Right allignment | =(ID7+ID8+ID9+ID10) TK |

- Call unit

Now BTCL divide one day to peak time and off peak time. The following table indicates how to calculate the call unit.

| Call type | Time segment | Call unit |
| --- | --- | --- |
| BTCL-BTCL | Peak time(8:00 ~20:00) | 3 call unit/min |
| BTCL-BTCL | Off peak time(20:00 ~8:00) | 1 call unit/min |
| BTCL-other | 0:00~24:00 | 1 call unit/min |

If a call cross the time segment, it will be divided two parts and calculate separately according following table. If the conversion duration of call is several minutes and several seconds, the several seconds will calculate as one minute.

| Start time | End time | duration | Call type | Call unit |
| --- | --- | --- | --- | --- |
| 9:00:10 | 9:00:20 | 10 second | BTCL-BTCL | 3 call unit(round up) |
|  |  |  | BTCL-other | 1 call unit(round up) |
| 9:00:10 | 9:01:10 | 1 minute | BTCL-BTCL | 3 call unit |
|  |  |  | BTCL-other | 1 call unit |
| 7:59:20 | 8:02:20 | 3 minute | BTCL-BTCL | 1call unit(7:59:20~8:00:20, non peak time) + 6 call unit(8:00:20~8:02:20, peak time) = 7 call unit |
|  |  |  | BTCL-other | 3 call unit |
| 19:59:20 | 20:02:20 | 3 minute | BTCL-BTCL | 3call unit(19:59:20~20:00:20, peak time) + 2 call unit(20:00:20~20:02:20, non peak time) = 5 call unit |
|  |  |  | BTCL-other | 3 call unit |
| 19:40:00 | 20:05:00 | 25minutes | BTCL-BTCL | 60 call unit(peak time) + 5 call unit(off peak time,round up) = 65 call unit |
|  |  |  | BTCL-other | 25 call unit |

Maybe some subscriber can enjoy the BTCL-BTCL or BTCL-other discount, the final call unit equal to the original call unit multiply by (100-discount)*100, then half adjust. The discount is configured and the default value is 0. The ‘half adjust’ means that if the final call unit is 4.5, then adjust to 5 units. If the final call unit is 6.4, then adjust to 6 units.

- IDD CALL

IDD call should adopt the ‘IDD rate.xlsx’ attatched file.

- OTHER ISSUE

1, Fax service is treated as voice call.

2, Internet service is fixed rent monthly. Billing center will do it. We ignore it here.

3, There isn't holiday discount and other discount type except the discount rate configured for BTCL-BTCL and BTCL-others. (for example: 600TK for disabled/freedom fighter will be processed in billing center. We ignore it here)

4, VAS(value-added service) are free.

5, There isn’t package.

- Number Plan

BTCL distinguish call to BTCL-BTCL, BTCL-OTHERS and IDD. There are 658 PSTN switchs in BTCL network and all the call between these PSTN switchs are BTCL-BTCL. The prefix list of BTCL is found in attatched file ‘ent_btcl_number_segment.xls’. The attatched file ‘exchange list.xls’ is only for reference.

If the caller and called are centrex user and number are in the range from 5000 to 7999, the call is local call. If caller or called is not centrex user, the call is ordinary call.

If the called is short code that are in the attatched file, the call is local call.

If the called number begin with ‘b55b’ or ‘b21b’, the call is local call.

The call that its prefix of called number begin with 00 & 012 is named ‘IDD call’

When the called number start with prefix described in below table and attatched file, the call is BTCL-OTHERS.

| ID | NAME | SHORT_NAME | PREFIX | OPERATOR_TYPE |
| --- | --- | --- | --- | --- |
| 67 | Dhaka Phone | DhakaPhone | 066 | PSTN |
| 68 | NationalPhone | NationalPhone | 037 | PSTN |
| 69 | Onetel | Onetel | 0644 | PSTN |
| 70 | Peoples Tel | Peoples Tel | 038 | PSTN |
| 72 | Telebarta | Telebarta | 036 | PSTN |

- PROCESSING STEP

1, Check whether the valid_indicator field of CDR is 0, if not, drop this CDR to invalid CDR directory and process next CDR;

2, Check whether the free_indicator fuekd of CDR is 1, if not, drop this CDR to free CDR directory and process next CDR;

3, Check whether the charge_party_indicator of CDR is 1, if not, drop this CDR to other CDR directory and process next CDR;

4, Calculate the call unit and call count according to call type, then accumulate the call unit to the record of caller number. All the record of subscriber will clear before generate the last month file. In the final CSV file, one subscriber only have one record.

#### Encoding

Encoding is to output CDR files as per downstream system’s required format.

Mediation and billing sub system will ouput the CSV format to billng center. The CSV file name should be rODTS_ + month + year + .csv. For example: ‘rODTS_Jan 2014.csv’.

The file content should be like following:

#### Uploading

Mediation and billing sub system needs to push the cdr’s to Billing Center through FTP protocol

| Property | Property value |
| --- | --- |
| Local File Process Mode | Backup |
| File List Sort Mode | ModifyTimeSort |
| Transmission Mode |  |
| Compress Upload Mode | Not Compress and Upload Directly |
| Control Mode | Periodical |
| Control timing | 60 second |
| Login IPAddress | Will be provided during integration |
| Login Username | Will be provided during integration |
| Login Password | Will be provided during integration |
| Port No. | Will be provided during integration |
| Login Password | Will be provided during integration |
| Remote File Path | Will be provided during integration |
| Remote File Storage Mode |  |

## Maintainance and Configuration

### Configuration

#### System configuration

Mediation and billing sub system provide system configuration fuction.

FTP parameters used for connect IGWB and billing center is configurable.

CDR processing and storage directory are configurable.

The number segment can be configurable. Up to 10 number segments.

#### Charging configuration

Peak time and off peak time is configurable.

Call unit is configurable.

The areacode and prefix of BTCL-OTHERS is configurable.

The short code are configurable.

The centrex number range is configurable. Now it is from 5000 to 7999.

IDD rate of every IDD prefix which begin with ‘00’ or ‘012’ should be configurable.

The discount for BTCL-BTCL and BTCL-other should be configurable. Two option will be provided and one is for all subscriber, the other option is for single subscribe. The default value is 0.

The line rent should be configurable. Like discount, two option will be provided. One is for all subscriber and other option is for single subscriber. The default value is 160.

#### Error CDR detection configuration

The condition for detecting error CDRs is configurable. We can configure several conditions that the amount is up to 10. The conditions have its sequence number. Mediation and billing sub system detect CDRs according to the sequence number.

### Maintainence

#### Browsing CDR

Mediation and billing sub system provide the function of browsing CDRs. We can browsing the CDR according to subscriber number or time segment.

#### Manually Processing CDR

If the mediation and billing sub system broke down or reset manually, maybe we need to charging all the CDRs of this month again. We can manually click the button to processing CDRs of all the subscriber or specified subscriber again according to specified time(month).

#### Statistic

Biling sub system can statistic the amount of received file, processed file and error file.

It can report the amount of call(BTCL-BTCL, BTCL-OTHERS and IDD), duration for each subscriber

#### Redundancy

When IGWB1 broke down, mediation and billing sub system can receive CDRs file from IGWB2.

If the mediation and billing sub system broke down or reset manually, it can process CDRs continually

## Performance Requirment

#### Performance requirment

Mediation and billing sub system can process 1M(1000,000) CDR per day.

Mediation and billing sub system can support 300,000 subscriber.

#### storage requirment

Mediation and billing sub system can store CDRs of two months.

Mediation and billing sub system should support RAID5.

## Log

Mediation and billing sub system can provide log function that help maintain system quickly. No option should be there to delete log. Enough space for storing log data of 2 year.